# Standing Desk Presence Tracker

## Overview

This project uses a Raspberry Pi with sensors to detect:

- Whether a user is present at their desk

- Whether the desk is in a sitting or standing position

Data is written to InfluxDB and visualized in Grafana.

# 1. Hardware Requirements

## Components

- Raspberry Pi (my example uses a Pi 4; other models should work also)

- HC-SR501 PIR Motion Sensor

- HC-SR04 Ultrasonic Sensor

- Jumper wires

- 1kΩ and 2kΩ resistors (for voltage divider on ECHO)

# 2. Wiring

## HC-SR501 (PIR)

- VCC → 5V (Pin 2 or 4)

- GND → Ground (Pin 6)

- OUT → GPIO17 (Pin 11)

## HC-SR04 (Ultrasonic)

- VCC → 5V

- GND → Ground

- TRIG → GPIO23 (Pin 16)

- ECHO → GPIO25 (Pin 22) via voltage divider

### Voltage Divider (IMPORTANT)

- ECHO → 1kΩ → GPIO25

- GPIO25 → 2kΩ → GND

This protects the Pi from 5V input.

# 3. Software Prerequisites

## Update system

sudo apt update && sudo apt upgrade -y

## Install Python packages

sudo apt install python3-pip -y
pip3 install RPi.GPIO requests

## Install InfluxDB (v1.8)

sudo apt install influxdb -y
sudo systemctl enable influxdb
sudo systemctl start influxdb

## Create database

influx
CREATE DATABASE room_monitor
exit

# 4. Sensor Test Scripts

## HC-SR501 PIR Test Script

# Detect motion and print timestamp

# Usage: ./hc-sr501-test.py

----

#!/usr/bin/python3

import RPi.GPIO as GPIO

import time

PIR_PIN = 17

GPIO.setmode(GPIO.BCM)

GPIO.setup(PIR_PIN, GPIO.IN)

print("PIR test running (Ctrl+C to stop)")

print("Stabilizing (30 sec)...")

for i in range(6):

time.sleep(5)

print(f"{(i+1)*5} seconds elapsed")

print("Ready")

motion_state = False

try:

while True:

current_state = GPIO.input(PIR_PIN)

if current_state == 1 and not motion_state:

print(f"{time.strftime('%H:%M:%S')} - MOTION DETECTED")

motion_state = True

elif current_state == 0 and motion_state:

print(f"{time.strftime('%H:%M:%S')} - NO MOTION")

motion_state = False

time.sleep(0.1)

except KeyboardInterrupt:

print("\nStopped")

finally:

GPIO.cleanup()

----

Expected:

<timestamp> - MOTION DETECTED

<timestamp> - NO MOTION

## HC-SR04 Distance Sensor Test Script

# Basic distance test – hc-sr501-test.py

# Usage: ./hc-sr04-test.py [repeat interval]

----

#!/usr/bin/python3

import RPi.GPIO as GPIO

import time

PIR_PIN = 17

GPIO.setmode(GPIO.BCM)

GPIO.setup(PIR_PIN, GPIO.IN)

print("PIR test running (Ctrl+C to stop)")

print("Stabilizing (30 sec)...")

for i in range(6):

time.sleep(5)

print(f"{(i+1)*5} seconds elapsed")

print("Ready")

motion_state = False

try:

while True:

current_state = GPIO.input(PIR_PIN)

if current_state == 1 and not motion_state:

print(f"{time.strftime('%H:%M:%S')} - MOTION DETECTED")

motion_state = True

elif current_state == 0 and motion_state:

print(f"{time.strftime('%H:%M:%S')} - NO MOTION")

motion_state = False

time.sleep(0.1)

except KeyboardInterrupt:

print("\nStopped")

finally:

GPIO.cleanup()

----

# 5. Core Logic Explanation

## Presence Detection (PIR)

- PIR detects motion

- Updates last_motion_time

- If no motion for X seconds → user is “Away”

## Desk Position (Ultrasonic)

- Measures distance to desk

- Threshold:

  - < 39 cm → Sitting

  - >= 39 cm → Standing

## State Machine

States:

- Away

- Present-Sitting

- Present-Standing

Logic: 1. If no motion → Away 2. If motion:

- Measure distance

- Compare to threshold

# 6. Data Model (InfluxDB)

Measurement:

desk_presence

Field:

value

Mapping:

- 0 = Away

- 1 = Sitting

- 2 = Standing

Write format:

desk_presence value=1

# 7. Heartbeat Design

The script writes:

- On state change

- Every 10 seconds (heartbeat)

Why:

- Enables accurate time tracking

- Simplifies Grafana queries

# 8. Grafana Setup

## Install Grafana

sudo apt install grafana -y
sudo systemctl enable grafana-server
sudo systemctl start grafana-server

## Access

- http://:3000

- Default: admin / admin

## Add InfluxDB Data Source

- URL: http://localhost:8086

- Database: room_monitor

- No auth (default)

# 9. Grafana Panels

## Current Status

Query:

SELECT last("value") FROM "desk_presence"

Visualization:

- Stat

- Value mappings: 0=Away, 1=Sitting, 2=Standing

## Timeline

Query:

SELECT "value" FROM "desk_presence" WHERE $timeFilter

Visualization:

- Time series

- Draw style: Step

## Time Distribution (Bar Gauge)

Queries:

- value=0

- value=1

- value=2

Transform:

- Multiply by 10

## Percentage (Pie Chart)

Same queries as above.

Visualization:

- Pie chart

- Show percent

# 10. Running the Script

chmod +x log-standing-time.py
./log-standing-time.py

# 11. Future Improvements

- Daily summaries

- Standing goals

- Session detection

- Alerts/notifications

# 12. Repository Structure

project/
├── scripts/
│   ├── main.py
│   ├── test_pir.py
│   ├── test_ultrasonic.py
├── docs/
│   └── README.md
├── grafana/
│   └── dashboard.json

# 13. Add to system startup

# Create: /etc/systemd/system/desk-monitor.service

Run: sudo nano /etc/systemd/system/desk-monitor.service

Include:

[Unit]

Description=Standing Desk Monitor

After=network.target

[Service]

ExecStart=/usr/bin/python3 /home/pi/standing-desk-detector/log-standing-time-influxdb.py

WorkingDirectory=/home/pi/standing-desk-detector

StandardOutput=inherit

StandardError=inherit

Restart=always

User=pi

[Install]

WantedBy=multi-user.target

Run:

sudo systemctl daemon-reexec

sudo systemctl daemon-reload

sudo systemctl enable desk-monitor

sudo systemctl start desk-monitor

sudo systemctl status desk-monitor

journalctl -u desk-monitor -f

# Summary

This system combines:

- PIR for presence

- Ultrasonic for desk height

- InfluxDB for storage

- Grafana for visualization

The heartbeat approach ensures accurate and simple analytics.